### Shared fixed step ray marcher

All implementations and measurements are done in the python framework “Rendervouz” [source]. The pipeline is built on a Vulkan compute-shader framework with direct GPU-buffer integration via PyTorch tensors. Each grid is implemented with a GLSL module that computes a concrete density value given a ray position (x,y,z). Camera position and cloud position are held constant and only the given data structure is swapped out.

For the ray-based experiments, transmittance along a ray is estimated with a fixed-step ray marcher common to all three representations:

- Intersect the ray with the volumes bounding box in object space to obtain a line segment spanning from t_min to t_max. If tMax is smaller or equal to zero or tMin is larger than tMax the ray points away from the volume and the calculation is stopped

- Advance a step counter t from a jittered offset in increments of a fixed step size s

- At each step query the extinction density at the current position (using one of the already described GLSL modules) and accumulate optical depth: tau += denstity * extinction_scale * s

- If tau exeeds a thertain threshold exit early or else return exp(-tau * scale)

It is important to acknowledge that apart from an early exit this ray marcher has fixed step size and does not use any sort of space skipping. While this is a good baseline measurement, Section X describes a ray marcher with space skipping.

The dense grid servers a

### Dense Grid Data Structure

The dense grid is the baseline data structure used to compare the two other structures. As such it is straightforward and uses no overhead in structure. The data is a row major flat tensor laying flat inside VRAM. While conceptually the tensor is of shape (D, H, W, OUTPUT_DIM), in the following implementations OUTPUT_DIM will always equal a single floating-point number representing the density of the volume at this specific (x,y,z) coordinate. Computationally the simple dense grid relies on strides to calculate a coordinate in [-1,1]^3 to a given index:

- First the coordinate (current sampling position) is mapped from [-1,1]^3 to [0,1]^3 with the simple formular (x,y,z) * 0.5 + 0.5

- The coordinate is then mapped from [0,1]^3 to [0, grid_size – 1]

- Flooring this new coordinate results in the exact position of the front, bottom, left voxel from the current sampling position. By adding (1,1,1) to this position, the back, top, right voxel is obtained

The dense grid servers a